Jetson reComputer 6.x Setup

On Ubuntu,

- Download NVIDIA SDK Manager on Ubuntu (at least 22.04 version) for Jetpack 6.x

- Start the SDK

- Flash Jetpack 6.x into the Jetson NVME using SDK

Enter Force Recovery Mode

Enter recovery mode on the reComputer Industrial board in order flash the device.

  - Connect a USB Type-C cable between USB2.0 DEVICE port and your PC.

  - Use a pin and insert into the RECOVERY hole to press recovery button and while holding this.

  - Connect the included 2-Pin Terminal block power connector to the power connector on the board and connect the included power adapter with a power cord to turn on the board.

  - Release the recovery button.

On Jetson,

- Update and install nvidia jetpack sdk CUDA

  - sudo apt update

  - sudo apt install nvidia-jetpack -y

  - sudo apt-get install cuda

- Install pip

  - sudo apt install python3-pip

  - sudo apt install nano

  - nano ~/.bashrc

  - Write at bottom script

export CUDA_HOME=/usr/local/cuda

export LD_LIBRARY_PATH=$LD_LIBRARY_PATH:/usr/local/cuda/lib64:/usr/local/cuda/extras/CUPTI/lib64

export PATH=$PATH:$CUDA_HOME/bin

  - source ~/.bashrc

- Install JTOP to check CUDA existence

  - sudo python3.10 -m pip install -u jetson-stats

  - sudo systemctl restart jtop.service

  - sudo reboot

- Download browser

  - sudo apt install epiphany-browser

  - sudo chown $USER:$USER $HOME/Downloads && chmod 755 $HOME/Downloads

- Download miniconda

  - mkdir -p ~/miniconda3

  - wget https://repo.anaconda.com/miniconda/Miniconda3-latest-Linux-aarch64.sh -O ~/miniconda3/miniconda.sh

  - bash ~/miniconda3/miniconda.sh -b -u -p ~/miniconda3

  - rm ~/miniconda3/miniconda.sh

  - source ~/miniconda3/bin/activate

  - conda init --all

- Download pylon software 25.07ARM 64 bit Debian Installation Package

  - https://www.baslerweb.com/en/downloads/software/3520605482/?downloadCategory.values.label.data=pylon

  - Extract downloaded file and change dir to pylon_*.deb files

  - sudo chmod 755 ./pylon_*.deb ./codemeter*.deb

  - sudo apt-get install ./pylon_*.deb ./codemeter*.deb

- Create env using conda with Python 3.10

  - conda create --name pygpu python=3.10

  - conda activate pygpu

  - pip install pypylon

  - pip3 install torch torchvision torchaudio --index-url https://pypi.jetson-ai-lab.io/jp6/cu126/

  - pip3 install "numpy<2"

  - python3 -c "import torch; print(f'PyTorch: {torch.__version__}'); print(f'CUDA Available: {torch.cuda.is_available()}'); print(f'CUDA Version: {torch.version.cuda}')"

  - pip install PySide6 opencv-python numpy

  - sudo apt install libdouble-conversion3 libopenblas-dev libxcb-xinerama0 libxcb-xfixes0 libxcb-shape0 libxcb-randr0 libxcb-cursor0

  - sudo apt install cmake

  - pip install rfdetr supervision

- Download VScode

  - https://code.visualstudio.com/download  .deb -> arm64

  - sudo dpkg -i code_1.104.1-1758154116_arm64.deb